拓扑图

测试目标：

- centos7/centos8,协议栈收发TCP性能

- 前端带业务，使用tcp传输数据的性能与资源占用

- 1份输入，复制2份输出模式下，单机最大支持输入多少流量

准备工作：

- 4根万兆光纤和8个万兆模块

- 准备三台服务器centos7

- 接线直连

配置工作

- 说明

以192.168.10.8为例，找两个没有配置ip的网卡，用来做聚合的物理网卡，192.168.10.9同理，192.168.10.28需要4个网卡配置2个聚合用来跟192.168.10.8和192.168.10.9之间互通

- 前端模拟服务器创建链路聚合team1

nmcli connection add type team con-name team1 ifname team1 config '{"runner":{"name":"roundrobin"}}'

team的四种模式（在这里我们使用轮训模式）

- broadcast (广播模式)

- activebackup (主备模式)

- roundrobin (轮训模式)

- loadbalance (负载均衡) LACP

- 将两张新网卡添加到这个team1中并查看网卡状态

nmcli connection add type team-slave con-name p6p1 ifname p6p1 master team1

nmcli connection add type team-slave con-name p7p2 ifname p7p2 master team1

[root@localhost ~]# nmcli device status

DEVICE  TYPE      STATE         CONNECTION

em1     ethernet  connected     em1

p6p1    ethernet  connected     p6p1

p7p2    ethernet  connected     p7p2

team1   team      connected     team1

- 配置team1的ip地址并修改BOOTPROTO=static,IPADDR=ip地址,PREFIX=24

Vim /etc/sysconfig/network-scripts/ifcfg-team1

DEVICE=team1

TEAM_CONFIG="{\"runner\":{\"name\":\"roundrobin\"}}"

DEVICETYPE=Team

BOOTPROTO=static

DEFROUTE=yes

IPV4_FAILURE_FATAL=no

IPV6INIT=yes

IPV6_AUTOCONF=yes

IPV6_DEFROUTE=yes

IPV6_FAILURE_FATAL=no

NAME=team1

UUID=7e5de1b0-f461-4bb6-bec8-e13c00204865

ONBOOT=yes

IPADDR=192.168.30.174

PREFIX=24

IPV6_PEERDNS=yes

IPV6_PEERROUTES=yes

- 重启网卡

systemctl restart network

- 查看聚合配置，p6p1与p7p2的mac地址相同

8: p7p1: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc mq state DOWN qlen 1000

link/ether e8:61:1f:18:ef:22 brd ff:ff:ff:ff:ff:ff

9: p7p2: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc mq master team1 state UP qlen 1000

link/ether e8:61:1f:13:6a:a2 brd ff:ff:ff:ff:ff:ff

10: team1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc noqueue state UP qlen 1000

link/ether e8:61:1f:13:6a:a2 brd ff:ff:ff:ff:ff:ff

inet 192.168.30.174/24 brd 192.168.30.255 scope global team1

valid_lft forever preferred_lft forever

inet6 fe80::ea61:1fff:fe13:6aa2/64 scope link

valid_lft forever preferred_lft forever

- 查看聚合网卡状态

[root@localhost network-scripts]# teamdctl team1 state

setup:

runner: roundrobin

ports:

p6p1

link watches:

link summary: up

instance[link_watch_0]:

name: ethtool

link: up

down count: 4

p7p2

link watches:

link summary: up

instance[link_watch_0]:

name: ethtool

link: up

down count: 5

- 配置路由

  - 192.168.10.9配置路由

route add –host 192.168.20.172 dev team1

  - 192.168.10.28配置路由

route add –host 192.168.20.171 dev team1

route add –host 192.168.30.174 dev team2

  - 192.168.10.8配置路由

route add –host 192.168.30.173 dev team1

- 测试

  - 在192.168.10.28上执行[root@localhost ~]# qperf

  - 在192.168.10.9上执行

[root@localhost ~]# qperf 192.168.20.172 tcp_bw

tcp_bw:

bw  =  1.79 GB/sec

  - bw = 1.79GB/sec达到效果