H3C交换机高可用测试

1.测试1

1.1.测试说明

使用技术：IRF堆叠

技术介绍：IRF 是Intelligent Resilient Framework的简称，IRF称之为智能弹性架构，是H3C自主研发的硬件虚拟化技术，它的核心思想是将多台设备通过IRF物理端口连接在一起，进行必要的配置后，虚拟化成一台“分布式设备”。使用这种虚拟换技术可以集合多台设备的硬件资源和软件处理能力，实现多台设备的协同工作、统一管理和不间断维护（简单说，IRF技术就是“硬件虚拟化技术”，将多个硬件设备虚拟化出一台更大的硬件设备）。

网络拓扑：

实验设备介绍

| 类型 | 型号 | 数量 |
| --- | --- | --- |
| 交换机 | H3C S6890 | 2 |
| 服务器 | 宁畅 R620 G40 | 2 |
| 网线 | 交换机管理网线 |  |
| 光纤 | 万兆光纤 |  |
| 光模块 | 10G 40G万兆光模块 |  |

1.1.1.IRF配置注意事项：

配置 IRF 需要用万兆接口，即 FGE 或 XGE 开头的接口

1.2.交换机配置

步骤一：查看设备编号 使用命令display irf查看设备的编号 即 MemberID 步骤二：修改设备编号 在 SWB 上配置 [SWB]irf member 1 renumber 2 //配置 IRF 设备的成员编号为2，需要确认输入 y [SWB]save //保存配置，需要确认输入 y [SWB]qu <SWB>reboot //重启，需要确认输入 y 重启之后在 SWB 上查看设备编号 使用命令 display irf查看设备编号 display irf configurationi显示本堆叠中所有设备的预配置信息 display interface brief查看接口 可以看出编号已经改为 2 步骤三：配置堆叠口 SWA [SWA]interface range FortyGigE 1/0/53 FortyGigE 1/0/54 //先关闭物理，再配置堆叠口 [SWA-if-range]shutdown [SWA-if-range]qu [SWA]irf-port 1/1 //创建堆叠口 [SWA-irf-port1/1]port group interface FortyGigE 1/0/53 //将53接口加入堆叠口 [SWA-irf-port1/1]port group interface FortyGigE 1/0/54 //将54接口加入堆叠口 [SWA]interface range FortyGigE 1/0/53 FortyGigE 1/0/54 //打开53、54接口 [SWA-if-range]undo shutdown 使用命令 display this 查看当前接口上的配置 保存激活,等待设备重启 [SWB]save [SWB]irf-port-configuration active 步骤四：检查 IRF 堆叠 SWB 重启后，查看设备编号，1为主设备，2位从设备，还可以发现 SWB 的设备名称和 SWA 的一样，因为已经堆叠成功，SWA 和 SWB 堆叠成一台，改变任意一个，另一个也会改变。 display irf topology查看本堆叠的拓扑信息

1.3.服务器配置

【Linux双网卡绑定】 1.1 先停止NetworkManager # service NetworkManager stop # chkconfig NetworkManager off 1.2 手工写绑定的配置ifcfg-bond0 vi /etc/sysconfig/network-scripts/ifcfg-bond0 DEVICE=bond0 TYPE=Ethernet ONBOOT=yes BOOTPROTO=none IPADDR=192.168.41.27 PREFIX=24 USERCTL=no 编辑第一块网卡配置 vi /etc/sysconfig/network-scripts/ifcfg-eth1 DEVICE=ens8f0 TYPE=Ethernet ONBOOT=yes BOOTPROTO=none MASTEB=bond0 slave=yes 编辑第二块网卡配置 vi /etc/sysconfig/network-scripts/ifcfg-eth2 DEVICE=ens8f1 TYPE=Ethernet ONBOOT=yes BOOTPROTO=none MASTEB=bond0 slave=yes 1.3 把绑定的网卡加载到内核去 vi /etc/modprobe.d/bonding.conf --设置绑定网卡别名 alias bond0 bonding options bond0 miimon=100 mode=1 参数说明： miimon=100：代表没100毫秒监测一次链路是否为通，如果这个网络不通，则使用第二个网络 mode=1：代表模式：1为一主一备 1.4 开机生效 vi /etc/rc.d/rc.local ifenslave bond0 eth1 eth2 chmod +x /etc/rc.d/rc.local 1.5 重启网络服务生效 service network restart 1.6 状态检查 ifconfig -a cat /proc/net/bonding/bond0

1.4.效果测试

| 测试项目 | 测试描述 | 效果 |
| --- | --- | --- |
| 网络联通状况 | 两台h3c交换机设备链型连接，做irf堆叠，两台服务器分别用两个网卡做bond1绑定管理地址互联分别连接到两个物理h3c交换机。当某一台物理交换机down掉。测试服务器业务是否中断 | 某一台交换机设备down掉，服务器业务口不会中断 |
| 高可用测试 | 某一台交换机下电，观察业务配置。 | IRF系统由多台成员设备组成，采用1:N备份，一台Master设备负责IRF系统的运行、管理和维护，多台Slave设备在作为备份的同时也可以处理业务，一旦Master设备故障，系统会迅速自动选举新的Master，转发流量和大部分业务都不会出现中断。 |
| 测试项目 | 测试描述 | 效果 |
| 网络联通状况 | 两个IRF组成的设备，配置改变 | 两个IRF组成的设备，控制层唯一，两个物理的交换机只有一个管理地址 |

2.测试2

2.1.测试说明

使用技术：DRNI堆叠

技术介绍：DRNI是一种跨设备链路聚合技术，将两台物理设备在聚合层面虚拟成一台设备来实现跨设备链路聚合，从而提供设备级冗余保护和流量负载分担。

网络拓扑：

Device A与Device B形成负载分担，共同进行流量转发，当其中一台设备发生故障时，流量可以快速切换到另一台设备，保证业务的正常运行。

图1 DRNI网络模型示意图

实验设备介绍

| 类型 | 型号 | 数量 |
| --- | --- | --- |
| 交换机 | H3C S6890 | 2 |
| 服务器 | 宁畅R620 G40 | 2 |
| 网线 |  |  |
| 光纤 |  |  |

2.1.1.DRNI配置注意事项：

2.2.交换机配置

(1) 配置Device A 1 # 系统配置。 2 <DeviceA> system‐view 3 [DeviceA] drni system‐mac 1‐1‐1 /配置drni的MAC地址 4 [DeviceA] drni system‐number 1 /配置drni系统的编号 5 [DeviceA] drni system‐priority 123 /配置drni的优先级 6 # 配置Keepalive报文的目的IP地址和源IP地址。 7 [DeviceA] drni keepalive ip destination 1.1.1.1 source 1.1.1.2 8 # 配置端口GigabitEthernet1/0/5工作在三层模式，并配置IP地址为Keepalive报文的源 IP地址。 9 [DeviceA] interface gigabitethernet 1/0/5 10 [DeviceA‐GigabitEthernet1/0/5] port link‐mode route 11 [DeviceA‐GigabitEthernet1/0/5] ip address 1.1.1.2 24 12 [DeviceA‐GigabitEthernet1/0/5] quit 13 # 配置Keepalive链路接口为保留接口。 14 [DeviceA] drni mad exclude interface gigabitethernet 1/0/5 15 # 创建二层聚合接口3，并配置该接口为动态聚合模式。 16 [DeviceA] interface bridge‐aggregation 3 17 [DeviceA‐Bridge‐Aggregation3] link‐aggregation mode dynamic 18 [DeviceA‐Bridge‐Aggregation3] quit 19 # 分别将端口GigabitEthernet1/0/1和GigabitEthernet1/0/2加入到聚合组3中。 20 [DeviceA] interface gigabitethernet 1/0/1 21 [DeviceA‐GigabitEthernet1/0/1] port link‐aggregation group 3 22 [DeviceA‐GigabitEthernet1/0/1] quit 23 [DeviceA] interface gigabitethernet 1/0/2 24 [DeviceA‐GigabitEthernet1/0/2] port link‐aggregation group 3 25 [DeviceA‐GigabitEthernet1/0/2] quit 26 # 将二层聚合接口3配置为IPP口。 27 [DeviceA] interface bridge‐aggregation 3 28 [DeviceA‐Bridge‐Aggregation3] port drni intra‐portal‐port 1 29 [DeviceA‐Bridge‐Aggregation3] quit 30 # 创建二层聚合接口4，并配置该接口为动态聚合模式。 31 [DeviceA] interface bridge‐aggregation 4 32 [DeviceA‐Bridge‐Aggregation4] link‐aggregation mode dynamic 33 [DeviceA‐Bridge‐Aggregation4] quit 34 # 分别将端口GigabitEthernet1/0/3和GigabitEthernet1/0/4加入到聚合组4中。 35 [DeviceA] interface gigabitethernet 1/0/3 36 [DeviceA‐GigabitEthernet1/0/3] port link‐aggregation group 4 37 [DeviceA‐GigabitEthernet1/0/3] quit 38 [DeviceA] interface gigabitethernet 1/0/4 39 [DeviceA‐GigabitEthernet1/0/4] port link‐aggregation group 4 40 [DeviceA‐GigabitEthernet1/0/4] quit 41 # 将二层聚合接口4加入分布式聚合组4中。 42 [DeviceA] interface bridge‐aggregation 4 43 [DeviceA‐Bridge‐Aggregation4] port drni group 4 44 [DeviceA‐Bridge‐Aggregation4] quit (2) 配置Device B 1 # 系统配置。 2 <DeviceB> system‐view 3 [DeviceB] drni system‐mac 1‐1‐1 4 [DeviceB] drni system‐number 2 5 [DeviceB] drni system‐priority 123 6 # 配置Keepalive报文的目的IP地址和源IP地址。 7 [DeviceB] drni keepalive ip destination 1.1.1.2 source 1.1.1.1 8 # 配置端口GigabitEthernet1/0/5工作在三层模式，并配置IP地址为Keepalive报文的源 IP地址。 9 [DeviceB] interface gigabitethernet 1/0/5 10 [DeviceB‐GigabitEthernet1/0/5] port link‐mode route 11 [DeviceB‐GigabitEthernet1/0/5] ip address 1.1.1.1 24 12 [DeviceB‐GigabitEthernet1/0/5] quit 13 # 配置Keepalive链路接口为保留接口。 14 [DeviceB] drni mad exclude interface gigabitethernet 1/0/5 15 # 创建二层聚合接口3，并配置该接口为动态聚合模式。 16 [DeviceB] interface bridge‐aggregation 3 17 [DeviceB‐Bridge‐Aggregation3] link‐aggregation mode dynamic 18 [DeviceB‐Bridge‐Aggregation3] quit 19 # 分别将端口GigabitEthernet1/0/1和GigabitEthernet1/0/2加入到聚合组3中。 20 [DeviceB] interface gigabitethernet 1/0/1 21 [DeviceB‐GigabitEthernet1/0/1] port link‐aggregation group 3 22 [DeviceB‐GigabitEthernet1/0/1] quit 23 [DeviceB] interface gigabitethernet 1/0/2 24 [DeviceB‐GigabitEthernet1/0/2] port link‐aggregation group 3 25 [DeviceB‐GigabitEthernet1/0/2] quit 26 # 将二层聚合接口3配置为IPP口。 27 [DeviceB] interface bridge‐aggregation 3 28 [DeviceB‐Bridge‐Aggregation3] port drni intra‐portal‐port 1 29 [DeviceB‐Bridge‐Aggregation3] quit 30 # 创建二层聚合接口4，并配置该接口为动态聚合模式。 31 [DeviceB] interface bridge‐aggregation 4 32 [DeviceB‐Bridge‐Aggregation4] link‐aggregation mode dynamic 33 [DeviceB‐Bridge‐Aggregation4] quit 34 # 分别将端口GigabitEthernet1/0/3和GigabitEthernet1/0/4加入到聚合组4中。 35 [DeviceB] interface gigabitethernet 1/0/3 36 [DeviceB‐GigabitEthernet1/0/3] port link‐aggregation group 4 37 [DeviceB‐GigabitEthernet1/0/3] quit 38 [DeviceB] interface gigabitethernet 1/0/4 39 [DeviceB‐GigabitEthernet1/0/4] port link‐aggregation group 4 40 [DeviceB‐GigabitEthernet1/0/4] quit 41 # 将二层聚合接口4加入分布式聚合组4中。 42 [DeviceB] interface bridge‐aggregation 4 43 [DeviceB‐Bridge‐Aggregation4] port drni group 4 44 [DeviceB‐Bridge‐Aggregation4] quit (3) 配置Device C 1 # 创建二层聚合接口4，并配置该接口为动态聚合模式。 2 <DeviceC> system‐view 3 [DeviceC] interface bridge‐aggregation 4 4 [DeviceC‐Bridge‐Aggregation4] link‐aggregation mode dynamic 5 [DeviceC‐Bridge‐Aggregation4] quit 6 # 分别将端口GigabitEthernet1/0/1～GigabitEthernet1/0/4加入到聚合组4中。 7 [DeviceC] interface range gigabitethernet 1/0/1 to gigabitethernet 1/0/4 8 [DeviceC‐if‐range] port link‐aggregation group 4 9 [DeviceC‐if‐range] quit 验证配置 1 # 查看Device A上分布式聚合的Keepalive报文信息。 2 [DeviceA] display drni keepalive 3 Neighbor keepalive link status: Up 4 Neighbor is alive for: 104 sec, 16 ms 5 Last keepalive packet sending status: Successful 6 Last keepalive packet sending time: 2017/03/09 10:12:09 620 ms 7 Last keepalive packet receiving status: Successful 8 Last keepalive packet receiving time: 2017/03/09 10:12:09 707 ms 9 Distributed relay keepalive parameters: 10 Destination IP address: 1.1.1.1 11 Source IP address: 1.1.1.2 12 Keepalive UDP port : 6400 13 Keepalive VPN name : N/A 14 Keepalive interval : 1000 ms 15 Keepalive timeout : 5 sec 16 Keepalive hold time: 3 sec 17 以上信息表明Device A和Device B设备间无故障。 18 # 查看Device A上IPP口和DR口的摘要信息和详细信息。 19 [DeviceA] display drni summary 20 Global consistency check : SUCCESS 21 Inconsistent type 1 global settings: 22 IPP IPP ID State 23 BAGG3 1 UP 24 DR interface DR group ID State Check result Type 1 inconsistency 25 BAGG4 4 UP SUCCESS /UP代表成功 26 [DeviceA] display drni verbose 27 Flags: A ‐‐ Home_Gateway, B ‐‐ Neighbor_Gateway, C ‐‐ Other_Gateway, 28 D ‐‐ IPP_Activity, E ‐‐ DRCP_Timeout, F ‐‐ Gateway_Sync, 29 G ‐‐ Port_Sync, H ‐‐ Expired 30 IPP/IPP ID: BAGG3/1 31 State: UP 32 Local state/Peer state: ABDFG/ABDFG 33 Local Selected ports Index: 1, 2 34 Peer Selected ports Index: 1, 2 35 36 DR interface/DR group ID: BAGG4/4 37 State: UP 38 Local state/Peer state: ABDFG/ABDFG 39 Local Selected ports Index: 16387, 16388 40 Peer Selected ports Index: 32771, 32772

2.3.服务器配置

配置bond 创建bond接口，在 /etc/sysconfig/network-scripts/ 目录中创建名为 ifcfg-bondN 的文件，使用接口号码替换 N，比如 0 1.### 创建bond接口 # vim /etc/sysconfig/network-scripts/ifcfg-bond0 DEVICE=bond0 //和文件名中的ifcfg-bond0中的bond0是一致的。 NAME=bond0 //设备名字，为了NetworkManage方便管理，和DEVICE保持一致 TYPE=Bond //TYPE要设置成Bond，表示该接口为bond接口 BONDING_MASTER=yes IPADDR=192.168.41.27 PREFIX=24 ONBOOT=yes BOOTPROTO=none BONDING_OPTS="mode=6 miion=100" //bond选项，引号里面的内容，需要配置，这里配置了之后，就不需要配置 2.### vim /etc/sysconfig/network-scripts/ifcfg-ens8f1 DEVICE=ens8f1 ONBOOT=yes HOTPLUG=no NM_CONTROLLED=no PEERDNS=no MASTER=bond0 // SLAVE=yes // BOOTPROTO=none 3.验证、查看bond ### 查看bond0的绑定结果 [root@control1 network-scripts]# cat /proc/net/bonding/bond0 Ethernet Channel Bonding Driver: v3.7.1 (April 27, 2011) Bonding Mode: fault-tolerance (active-backup) Primary Slave: None Currently Active Slave: ens8f1 MII Status: up MII Polling Interval (ms): 100 Up Delay (ms): 0 Down Delay (ms): 0 Slave Interface: ens8f1 MII Status: up Speed: 10000 Mbps Duplex: full Link Failure Count: 0 Permanent HW addr: b8:ca:3a:6c:49:68 Slave queue ID: 0 Slave Interface: ens8f2 MII Status: up Speed: 10000 Mbps Duplex: full Link Failure Count: 0 Permanent HW addr: b8:ca:3a:6c:49:6a Slave queue ID: 0

2.4.效果测试

| 测试项目 | 测试描述 | 效果 |
| --- | --- | --- |
| 网络联通状况 | 服务器断开其中一根线，另外一根线是否可以保证通信 | 服务器网卡bond6，可以保证网络通信不中断 |
| 高可用测试 | 交换机设备级冗余备份 | 测试失败，没有达到设备高可用测试效果 |